Refit the ancillary belt (2), checking that the belt is correctly fitted in the grooves of the different pulleys and if re-using the old belt, ensure that it is refit in the correct fitting direction. Remove the peg [0188-Q1]; Using tool [0188-Z]. Refit: - The front right mud guard - The front right wheel - The protective panel under the engine

Part Two: Timing Belt

Tools Used: Refer to Previous Post

| During removal: Plug the fuel low pressure supply pipes; Using the plugs [1617-J]. - Uncouple the unions of the fuel low pressure circuit (at "a"). - Unclip and move aside the manual priming pump (1). - Remove the mounting of the diesel fuel manual primer pump (2) ; By means of a screwdriver (at "b"). | Remove the air union (3) CAUTION: Use a rubber shim of sufficient surface area to avoid distorting the engine oil sump (Risk of obstruction of the oil pump suction strainer.) Support the engine using a jack and a soft block (Timing gear side). Remove: - The mountings (4) - The bolts (7) - The engine supports (5) and (6) |
| --- | --- |
| Disconnect the connector of the engine speed sensor (at "c"). Release and move aside the electrical harness (8). Remove: - The pulley cover (11) - The bolts (10) - The top timing cover (9) | CAUTION: It is essential to rotate the engine by means of the bolt (12) and in the clockwise direction only. Peg: - The camshaft pulley ; Using tool [0188-M] - The flywheel ; Using tool [0188-Y] Lock the flywheel using the retainer [0188-F]. |
| CAUTION: The target (16) must show no trace of damage and no magnetic source must go near it. CAUTION: Use of an impact wrench is prohibited (Risk of the bolt (12) breaking in the crankshaft) Remove: - The bolt (12) and its spacer - The crankshaft pulley (15) - The engine speed sensor target (16) - The 3 bolts (14) - The timing cover (13) Clean the crankshaft thread with a M14x150 thread cutter. | Unscrew nut (17). Detension the dynamic tensioner roller (18). Remove the timing belt (19). |
| Verification Check that the following components rotate freely (Absence of play and points of resistance): - Dynamic roller tensioner (18) - Roller tensioner (20) - Crankshaft pulley (21) on keyways - Water pump pulley (22) Visually check: - That there is no trace of an oil leak - That there are no coolant leaks - That there is no rubbing on the timing covers CAUTION: Replace any defective parts. | Dynamic Roller Tensioner CAUTION: The operations for replacing and adjusting the tension of the dynamic tensioner roller (18) are performed with the aid of a mirror. Replacement CAUTION: Do not release the new dynamic tensioner roller (18) without having refitted the timing belt (19). Remove: - The nut (17) - The stud (23) - The dynamic roller tensioner (18) CAUTION: The dynamic tensioner roller (18) should be secured in the cylindrical pin (25). Refit: - The dynamic roller tensioner (18) (new) (Locked) - The stud (23) - The nut (17) ; Finger tighten - Align the eccentric (24) on the axis "d" ; Using a hexagonal spanner |
| C5 X7 DW10CTED4 Timing Belt 9.PNG (181.25 KiB) Viewed 229 times |  |
| If Re-Used: CAUTION: The dynamic tensioner roller (18) should be secured in the cylindrical pin (25). Align the eccentric (24) on the axis "d" ; Using a hexagonal spanner. | Refitting: CAUTION: Adhere to the tightening torques. Pre-Tightening The Belt CAUTION: Always replace the timing belt (19). - Rotate the crankshaft pulley (21) on its cotter pin; Centre using the tool [0188-AH]. - Fit the timing belt (19) on the camshaft pinion ; Immobilise the pinion ; Using tool [0188-AD]. - Position the timing belt (19) on the pulleys in the order indicated ; Span ("e") of the belt correctly tensioned. CAUTION: New dynamic roller tensioner (18): Take away the locking pin. |
| Position of the index; State of tension of the timing belt. "f"; Dynamic roller tensioner (18) at maximum timing belt tension. "g"; Dynamic roller tensioner (18) at nominal timing belt tension. "h"; Dynamic roller tensioner (18) at minimum timing belt tension. Apply a maximum tension to the timing belt (19): - Do up the nut (17) by hand - Rotate the eccentric (24) anti-clockwise - Bring the index to position "f" - Tighten the nut (17) N.B. : The eccentric (24) is close to the horizontal. Remove the tools [0188-AD], [0188-AH]. | CAUTION: Systematically replace the screw (12); Do not go beyond 5 minutes between pre-tightening and final tightening of the bolt . Refit: - The timing cover (13) - The 3 bolts (14) - The engine speed sensor target (16) - The crankshaft pulley (15) - The bolt (12) and its spacer Remove the tools [0188-F], [0188-M], [0188-Y]. Turn the engine 6 rotations in the clockwise direction. Check the position of the timing belt (19). Peg the assembly with tools [0188-F], [0188-M], [0188-Y]. Slacken the bolt (12) in order to permit the crankshaft pulley to rotate on its keyways. |

C5 X7 DW10CTED4 Timing Belt 9D.PNG (162.64 KiB) Viewed 229 times

Final Tension CAUTION: When detensioning the dynamic tensioner roller, do not go past the index "g". Otherwise, you must restart the pretensioning operation. Apply a nominal tension to the timing belt: - Slacken the nut (17) while maintaining the eccentric (24) in position - Slowly rotate the eccentric (24) in the clockwise direction - Bring the index to position "g" - Tighten to the specified torque: The nut (17) Retighten the bolt (12) to the correct torque. Remove the tools [0188-F], [0188-M], [0188-Y]. Refit the pulley cover (11) ; Using rubber lubricant CAUTION: Check the correct positioning of the pulley cover by pulling gently on the lip.